Scheme of Work - Year 7

| Total Lessons | 5 |  | Design & Make – Key Stage 3 |
| --- | --- | --- | --- |
| Lesson Time | 1hr |  | Biscuit Cutter Project |

Objectives This project will give pupils the opportunity to work with MDF to produce a pattern which can be used with a vacuum forming machine and styrene sheet to produce both the cutter and by working in groups the tray or insert for the biscuit box.

Main Key Experiences & Essential Outcomes Children will be taught :- - About symbols that could be used for spring. - How to mark out the material. - How to shape MDF to produce a suitable pattern based on a spring symbol or shape. - How to work with styrene sheet and about thermoplastics (CF thermosetting plastics). - How to use the vacuum forming machine safely. - To work in groups for certain parts of the project for mutual benefit. # Children will be able to:- - Produce a paper template and then use it to mark out the final pattern. - To cut out the MDF carefully including the draft allowing ease of removal following molding. - Work safely using a range of tools, especially coping saw, sanding machine, abrasive papers (rubbing board). - Finish the work using appropriate abrasives to achieve an effective pattern, realise the need for a handle on the cutter and add one. - Work with the styrene sheet by vacuum forming followed by careful trimming, - Finally use the biscuit cutter and then fit all the parts together to present a complete biscuit package. - Add to their design folder showing the stages in the design process which led to the biscuit cutter production- brief, initial ideas including symbols for spring and biscuit shapes, final design drawing, the paper template, as well as the actual artifact. # Children will begin to understand: - The importance of working accurately if the biscuit cutter is to be effective as well as attractive. - The difference between a thermoplastic such as styrene and a thermosetting plastic such as Bakelite. - How the vacuum forming machine works (very simply), and how to use it safely. - The importance of working safely especially when working with sharp tools and machines - the danger of dust when using MDF. - How to produce an effective pattern for use on the vacuum forming machine.

| Programmes of Study 1a, 1b, 2a, 2b 3d, 3h, 4a, 4b, 4c, 4g, 4h, 4i 4j, 5a, 5b, 5d, 5e 8a, 8b 9a, 9b 10a, 10b, 10c |  |  | Assessment Possible levels: 3-5 - Planning ahead when making - Working with materials - Correct use of materials - Accuracy / Finish - Evaluating - End of project test |  |  |
| --- | --- | --- | --- | --- | --- |
| Differentiation Worksheets provided In the main by outcome (following instructions, accuracy and finish). Additional research can be introduced for extension activities. Isometric drawing of cutter. |  |  |  |  |  |
| RESOURCES |  |  |  |  |  |
| Equipment Tenon saw Bench hook Coping saw Rasps Files Glass paper | Vacuum forming machine Sanding board |  | Materials MDF blanks 9x80x80mm Card 80x80mm Styrene sheet |  |  |
| Literacy Introduce pupils to the following new words: Thermoplastic, Draft, Vacuum, Jig, Template, Pillar drill, Cross filling, Draw Filing, Medium Density Fibreboard. |  |  |  |  |  |
| Literacy |  | Oral, reading, writing. |  |  |  |
| Numeracy |  | Measuring, marking out |  |  |  |
| ICT opportunities |  | Work sheets |  |  |  |
| Personal qualities |  | Recapping |  |  |  |
| Interpersonal skills |  | Working with each other. |  |  |  |
| Links to other subjects |  | Science, Art, Maths, English |  |  |  |

| The programme |  |
| --- | --- |
| Lesson | Details |
| One | - Introduction to project: What it is, materials used, processes and safety. - Demonstration: Cutting card template. - Pupils: Specification, ideas for theme / shapes. (use worksheets provided) - Four ideas, Survey and Evaluation of ideas . - Produce card template – transfer designs to MDF. Homework: Complete final design |
| Two | Review Homework - Demonstration: Cutting using Tenon saw and bench hook, Coping saw. Shaping using rasp and file. - Explain the importance of the 'DRAFT' angle when producing moulds. - Explain the importance of rounding the corners. - Pupils: Practical activity |
| Three | - Demonstration: Vacuum forming - Pupils: Practical activity Homework: Complete Vacuum forming sheet |
| Four | - Review Homework - Pupils: Continue practical activity . |
| Five | Review Homework - Demonstration: Trimming vacuum formed shape and finishing on sanding board. - Pupils: Complete practical and isometric drawing of cutter. - Test cutters - End of project test. Homework: Complete Word search Game |

Evaluation

Notes